# AUSTRALIAN NEW CAR ASSESSMENT PROGRAM (ANCAP)

# ASSESSMENT OF SEAT BELT REMINDER SYSTEMS

# QUESTIONNAIRE FOR VEHICLE MANUFACTURER

In accordance with EuroNCAP assessment procedures, Australian NCAP now assigns bonus points for seat belt reminder systems that comply with specifications prepared by EuroNCAP [protocol document pending].

This questionnaire is designed to assist in the assessment of seat belt reminder systems. Numbers in brackets refer to clauses in the EuroNCAP document.

In order to provide transparency in the assessment process, ANCAP reserves the right to make this information available to other parties.

| SEATING POSITION | DRIVER | FRONT PASSENGER | (ALL) REAR SEATS |
| --- | --- | --- | --- |
| Do you wish to have this seating position assessed? |  |  |  |
| A (1.1-1.3) Occupant detection. Is the seat provided with a system capable of detecting a small adult female (5%ile) and larger occupants? | (Optional) | (Mandatory) | (Optional) |
| B (1.1-1.3) Seat belt usage. Is the seat provided with a system to detect when the seat belt is being used? Does it detect when the seat belt is unreeled? (rather than just use of the buckle) | (Mandatory) (Preferred) | (Mandatory) (Preferred) | (Mandatory) (Preferred) |
| C (1.4 & 2.2) Vehicle Use. How does the system determine that the vehicle is in use? - Engine running for more than 60s (optionally after forward speed reaches 10km/h) - Forward speed more than 25km/h - Forward travel more than 500m (optionally after forward speed reaches 10km/h) | (Mandatory - at least one) |  |  |
| D (2.2) Intermediate alert. Is the system provided with an intermediate alert (e.g. text or voice message)? If YES: Please describe Does the main audio/visual signal start after one (or more) of the following? - not more than 30 seconds after intermediate alert - Speed 40km/h or more - Distance not more than 500m after the intermediate alert Are alternative methods of discouraging non-wearing of seat belts used? (eg radio inoperative, air conditioning inoperative). | (Optional) (Mandatory if YES - at least one) (Optional) |  |  |
| E (3.2) Visual signal. Describe the location and type of visual signal. i) Is it also visible to the front passenger? ii) Does the visual signal remain active for at least 90s? iii) Does the visual signal remain active whenever a seat belt in an occupied front seat is not used? | (Mandatory) (Optional - preferred) (Mandatory) (Preferred) | (Mandatory) (Optional - preferred) (Mandatory) (Preferred) |  |
| F (3.2) Audible signal. Describe the type of audible signal. i) Approximate dBA at maximum volume (measured between driver and passenger ears) ii) Does the audible signal sound for at least 90s? (a progressive increase is acceptable, quiet periods longer than 3s are not counted in the cumulative total) | (Mandatory) (Preferred) (Mandatory) | (Mandatory) (Preferred) 7 (Mandatory) |  |
| G (2.3) Rear seat. Does the rear seat system (if fitted) have the same visual and audible systems as the front seats (eg does it detect seat occupancy as well as seat belt use)? If NO: i) Describe the alternative method of informing the driver about the use of seat belts ii) Is the status signal active for at least 30s? |  |  | (preferred) (mandatory) |
| H (2.2) Change in use. Does the system reactivate (ie alarm triggers) if a seat belt is taken off while the vehicle is in use? | (Mandatory - visual and audible) | (Mandatory - visual and audible) | (Mandatory - status only) |
| I (4.2) Short term disconnection. Can the system be over-ridden for the purpose of one trip? If YES: - How is this done? (eg buckling and unbuckling within 10s) - Does the system reactivate if the ignition is off for more than 60s? | (Optional) (Mandatory) |  |  |
| J (4.3) Long term disconnection. Can the system be disconnected indefinately? If YES - Briefly, how is this done? - Are the instructions for disconnection provided with the vehicle (ie in owners handbook)? - Is the system active when delivered to the consumer? - How is the system reactivated? (must be simple) | (Optional) (Mandatory - NO) (Mandatory - YES) |  |  |

## Assessors report

Road test conducted? Y / N		Result:  Emergence =       dB(A)

| Bonus points awarded | Driver | Front Pass. | Rear seats |
| --- | --- | --- | --- |

Signed 				Name					Date

Comments and enquiries about this document to:

Michael Paine

Phone 02 94514879

email: mpaine@tpgi.com.au

Please note that the assessment protocol is being developed by EuroNCAP (www.euroncap.com) and policy questions should be directed to that organisation. This questionnaire is an interpretation of a draft document issued by EuroNCAP in November 2002.